Omkar Gholap | D15A | RollNo. 17

# Experiment No. 1

Aim: To study different types of wireless Technology

Wireless Technology

## 1. Wi-Fi (Wireless Fidelity)

- Range of Communication: Typically up to 100 meters (indoors). The range can extend with the use of Wi-Fi extenders or routers supporting higher frequencies like 5 GHz.

- Cost: Moderate. Wi-Fi routers and devices are widely available at various price points.

- Propagation Delay: Moderate. Wi-Fi operates on the 2.4 GHz and 5 GHz bands, which are relatively less prone to significant delays compared to lower-frequency systems like LoRa.

- Power Consumption: Moderate to high. Devices like smartphones, laptops, and routers consume a considerable amount of power.

- Throughput: High. Wi-Fi can support speeds ranging from 72 Mbps (Wi-Fi 4) up to 9.6 Gbps (Wi-Fi 6).

- Fanout: High. Wi-Fi can handle numerous devices simultaneously (hundreds to thousands in a managed environment like a Wi-Fi 6 network).

- Applications: Home/office networking, public Wi-Fi, streaming, smart home devices.

## 2. Bluetooth

- Range of Communication: Short-range, typically up to 100 meters (for Bluetooth 5.x), but often much shorter (10 meters) in practical use.

- Cost: Low. Bluetooth chips are inexpensive, and devices supporting Bluetooth are common.

- Propagation Delay: Low. Bluetooth has low latency, ideal for applications like audio streaming.

- Power Consumption: Low. Bluetooth Low Energy (BLE) is designed to use minimal power, making it suitable for IoT devices.

- Throughput: Low. Bluetooth throughput typically ranges from 1 Mbps to 2 Mbps (BLE can go higher, but still not comparable to Wi-Fi).

- Fanout: Moderate. Bluetooth can connect several devices in a network, but it’s not ideal for large-scale communication compared to Wi-Fi or Zigbee.

- Applications: Wireless audio (headphones, speakers), health devices (fitness trackers), IoT devices, peripherals (keyboards, mice).

## 3. Zigbee

Range of Communication: Short to medium range (10-100 meters), depending on the environment.

- Cost: Low. Zigbee modules are inexpensive and widely used in IoT devices.

- Propagation Delay: Low to moderate. Zigbee is designed for low-latency, low-power applications.

- Power Consumption: Very low. Zigbee is optimized for low power usage, often running on small batteries for extended periods (years in some cases).

- Throughput: Low. Zigbee offers a maximum throughput of around 250 kbps.

- Fanout: High. Zigbee supports mesh networking, which allows many devices to communicate with one another, enabling large networks in IoT scenarios.

- Applications: Home automation, industrial IoT, smart energy, healthcare, building management.

## 4. LoRa (Long Range)

- Range of Communication: Very long-range, up to 15-20 km in rural areas and several kilometers in urban environments.

- Cost: Low. LoRa devices are relatively inexpensive and used in applications like smart agriculture and IoT.

- Propagation Delay: High. Due to the low-frequency signals used (typically around 868 MHz and 915 MHz), LoRa can experience higher propagation delay.

- Power Consumption: Very low. LoRa devices are designed for low power consumption, which allows them to run for years on a single battery.

- Throughput: Low. LoRa supports data rates from 0.3 kbps to 27 kbps, depending on the settings.

- Fanout: Moderate. LoRa supports a limited number of devices per gateway in a wide area network setup, but it’s not as scalable as cellular networks.

- Applications: Smart cities, agriculture, environmental monitoring, asset tracking, long-range IoT networks.

## 5. 5G (Fifth Generation)

- Range of Communication: Moderate. The range varies greatly depending on the frequency used, from several kilometers in low-band (Sub-6 GHz) to a few hundred meters in high-band (mmWave).

- Cost: High. Infrastructure costs for 5G are significant, and devices supporting 5G are more expensive than those for 4G or Wi-Fi.

- Propagation Delay: Very low. 5G promises ultra-low latency with a typical delay of around 1 ms.

- Power Consumption: Moderate to high. 5G devices require more power than 4G or Wi-Fi but are optimized for high-performance, low-latency connections.

- Throughput: Very high. 5G can theoretically reach up to 20 Gbps, with real-world speeds typically ranging from 1 to 10 Gbps.

- Fanout: Very high. 5G is designed to support massive numbers of connected devices (including IoT) and large-scale fanout.

Applications: Mobile broadband, IoT, autonomous vehicles, smart manufacturing, VR/AR, telemedicine.

## 6. NFC (Near Field Communication)

- Range of Communication: Very short (typically less than 10 cm).

- Cost: Very low. NFC chips are extremely inexpensive, and they are often integrated into smartphones, smart cards, and payment systems.

- Propagation Delay: Very low. NFC has near-instantaneous communication with minimal delay.

- Power Consumption: Low. NFC typically uses passive power from the reader device, so the powered device is usually the one consuming energy.

- Throughput: Very low. NFC supports data transfer rates up to 424 kbps.

- Fanout: Low. NFC is designed for point-to-point communication, typically between two devices only.

- Applications: Mobile payments, access control, ticketing, authentication, data sharing.

## 7. Infrared (IR)

- Range of Communication: Short range, typically up to 10 meters.

- Cost: Low. IR transmitters and receivers are very inexpensive.

- Propagation Delay: Low. IR signals travel at the speed of light with negligible delay.

- Power Consumption: Low. IR devices consume very little power, especially in consumer electronics like remote controls.

- Throughput: Low. IR supports a data rate up to 4 Mbps, but this is typically lower than other wireless technologies.

- Fanout: Low. IR is mostly used for one-to-one communication, such as in remote controls.

- Applications: Remote controls, wireless data transfer, motion detection, medical thermometers, robotics.

## 8. Satellite Communication

- Range of Communication: Very long range. Satellite communication can cover the entire Earth, making it ideal for global communication.

- Cost: High. Setting up and maintaining satellite infrastructure is expensive, and devices can also be costly.

- Propagation Delay: High. The long distance between Earth and satellites results in noticeable latency, typically 500-600 ms for geostationary satellites.

- Power Consumption: High. Satellite communication requires significant power for both the satellite and the ground stations.

- Throughput: Moderate to high. Throughput can vary depending on the satellite system; typical commercial satellite services provide speeds from a few Mbps to several hundred Mbps.

Fanout: High. Satellites can handle large-scale communication for a vast number of devices globally.

- Applications: Global communication, broadcasting, weather forecasting, GPS, search and rescue.

## 9. Cellular Communication (4G/5G/Other Generations)

- Range of Communication: Moderate. Cellular networks can cover vast areas, typically from several kilometers in rural areas to a few hundred meters in urban areas, depending on the generation (4G, 5G).

- Cost: High. Infrastructure costs are substantial, and mobile data plans can add to the cost for end users.

- Propagation Delay: Moderate to low. 4G has latencies of 30-50 ms, while 5G can lower this to sub-10 ms in ideal conditions.

- Power Consumption: Moderate to high. Cellular devices like smartphones consume a decent amount of power, especially on 4G/5G.

- Throughput: High. 4G typically offers speeds up to 100 Mbps, while 5G can offer speeds from 1 to 20 Gbps, depending on network conditions.

- Fanout: Very high. Cellular networks are built for large-scale deployment, serving millions of users with high fanout.

- Applications: Mobile phones, mobile broadband, smart cities, connected vehicles, industrial automation, telemedicine.

| Technology | Range | Cost | Propagation Delay | Power Consumption | Throughput | Fanout |
| --- | --- | --- | --- | --- | --- | --- |
| Wi-Fi | 100 meters | Moderat e | Moderate | Moderate to high | High (up to 9.6 Gbps) | High |
| Bluetooth | 100 meters | Low | Low | Low | Low (up to 2 Mbps) | Moderate |
| Zigbee | 10-100 meters | Low | Low to moderate | Very low | Low (250 kbps) | High |
| LoRa | 15-20 km | Low | High | Very low | Low (up to 27 kbps) | Moderate |
| 5G | Several km | High | Very low | Moderate to high | Very high (up to 20 Gbps) | Very High |
| NFC | <10 cm | Very low | Very low | Low | Very low (424 kbps) | Low |

| Infrared | 10 meters | Low | Low | Low | Low (up to 4 Mbps) | Low |
| --- | --- | --- | --- | --- | --- | --- |
| Satellite | Global coverage | High | High | High | Moderate to high | High |
| Cellular (4G/5G) | Several km | High | Moderate to low | Moderate to high | High (4G: 100 Mbps, 5G: up to 20 Gbps) | Very High |