EXPERIMENT 6

AIM: Interfacing of component in project

THEORY:

Interfacing electronic components is a fundamental step in building any embedded system. In the context of this gas leakage detection system, component interfacing refers to the correct electrical and logical connections between various hardware parts—such as the MQ5 gas sensor, Arduino Uno, LED, and supporting passive components—so they work together as a functional unit.

The main objective of interfacing in this project is to enable smooth data flow from the sensor (input) to the microcontroller (Arduino), and then control the output devices (LEDs, buzzers, etc.) based on the logic programmed.

In this project:

- The MQ5 sensor is connected to the analog input pin A0 of the Arduino to send continuous gas concentration data in the form of voltage.

- The LED is connected to a digital output pin (e.g., pin 13) to receive ON/OFF signals depending on whether the gas level has crossed the defined threshold.

- A 220Ω resistor is placed in series with the LED to limit current and prevent damage.

- A breadboard and jumper wires are used to make all temporary connections safe and organized for prototyping and testing.

Understanding how each component is connected and how it interacts with others is essential for ensuring the reliability and functionality of the overall system.

Component Interfacing Summary:

| Component | Interfacing Details & Pins | Notes |
| --- | --- | --- |
| MQ5 Sensor | VCC → 5V (Arduino), GND → GND, AOUT → A0 (Analog input pin) | Sends analog voltage proportional to gas concentration |
| Arduino Uno | USB → PC for code & power, A0 for MQ5 input, D13 for LED output | Controls logic, processes data, and triggers output signals |
| LED | Anode → D13 (via 220Ω resistor), Cathode → GND | Turns ON/OFF based on gas level |
| Resistor | 220Ω between D13 and LED Anode | Limits current to protect the LED |
| Breadboard | Central platform for making connections | Allows for easy testing and adjustments |
| Jumper Wires | Used for all pin connections | Ensures secure and flexible wiring between components and Arduino |

CONCLUSION:

The interfacing of components in this project was successfully achieved, ensuring seamless communication between the sensor, microcontroller, and output indicators.

Key takeaways:

- Proper power supply and grounding are crucial for component functionality.

- Analog signals from the MQ5 sensor can be effectively read and processed by the Arduino.

- Digital output signals from the Arduino can successfully control external devices like LEDs.

- Using passive components (like resistors) appropriately helps protect sensitive components.